Impossible Biomimetic Synthesis: AI Reasoning in Engineering Design

Abstract

We designed a hierarchical, constrained multi-agent synthesis framework to study how a language model reasons when tasked with engineering design. The system instructed a language model to synthesize a hydraulic rock breaker with a pistol shrimp. Given the deep incompatibility between these systems, any coherent synthesis of the two was extremely unlikely. This deliberate adversarial framing however created conditions for observing where model reasoning succeeds and where it degrades. The framework decomposed mechanical systems across six levels of abstraction, enforced provenance tracking against source specifications, and validated outputs against 22 physics and structural constraints. The system produced seven discrete designs of varying technical complexity. We observed design decisions reflecting genuine synthesis and creative problem-solving, but also spurious rationalizations and difficulty sustaining multi-step inference and managing long-range dependencies. Overall, out results suggest that language models can effectively navigate constrained design spaces, although nothing in our findings indicates they can perform the radical restructuring and reconceptualization underlying paradigm-shattering breakthroughs. AI's most productive contribution to engineering may be in accelerating the systematic exploration and incremental optimization that support the engineering process, while the breakout insights that drive revolutionary advances still exceed the capabilities of current architectures.

Introduction

Amid the hyperbole and messianic claims, one application of AI continues to loom large and command serious attention – that of scientific discovery. While some diminish language models as glorified token prediction, others view them as sophisticated versatile learning architectures. Hardliners in this latter camp argue that all cumulative intellectual progress is merely scaffolding for the arrival of more advanced computational machinery.

Scientific discovery is elusive and amorphous, and efforts to systematically study it are fraught. But the same creative energy underlying discovery also animates a different and more empirically constrained form of invention: engineering design.

Expanding new frontiers places ongoing demands on the engineer. Although this creative process resists formulaic description, it is mediated by imagination, intuition, and experience. A natural question thus arises: can creative technical work be reduced to algorithmic search - can an AI system replicate or approximate the human act of engineering design?

As a test of this proposition, we designed and implemented a system of technical specification, product design, and constraint and requirement validation – all fully executed by a language model. This was structured as a hierarchical, constrained multi-agent synthesis framework with LLM-based provenance and physics validation. In our architecture, a language model iteratively moves across different levels of design abstraction, selects components from a closed library of primitives, and satisfies provenance tracking and physics constraints. Grounding the model output in these requirements enforces coherence, restricts erroneous invention, and prevents violations of the laws of physics.

Our experiment was framed as a synthesis task: we instructed a language model to synthesize an existing industrial tool (hydraulic rock breaker) with a biological organism (pistol shrimp) to generate a novel hybrid mechanical design. This specific pairing was intentional. Although both possess a striking mechanism, the rock breaker and pistol shrimp use disparate energy systems (percussion vs cavitation), work in different mediums (air vs water), and operate at energy scales separated by orders of magnitude (kilowatts vs. milliwatts). This adversarial pairing was almost certain to result in failure – the AI faced an impossible task of reconciling fundamentally incompatible systems.

Counterintuitively, this adverse pairing was advantageous for our work. Language models, when forming hypotheses, frequently resort to handwaving or superficially plausible explanations that are ill-conceived or incoherent. An adversarial pairing combined with robust validation offers a useful stress test: as the model strains to find unorthodox solutions, we gain visibility into parts of the pipeline where performance degrades. Although we know in advance that the model will fail, knowing exactly how it fails is instructive.

Our results yielded four central findings: residual spurious reasoning despite constrained validation, occasional moments of sharply creative problem-solving, weaknesses in managing long-term dependencies and multi-step inference, and diversity in the exploration of designs. These findings inform a broader discussion on the nature of creativity in technical disciplines and the role of AI architectures in supporting engineering and scientific discovery.

Methodology

Case Selection

Biomimetic design – leveraging biological principles in building mechanical objects – has long influenced advances in robotics and engineering. This motivated the pairing of an industrial tool with a biological organism. It enables surveying natural selection to uncover new sources of novelty in mechanical design.

The specific pairing of the hydraulic rock breaker and pistol shrimp was driven both by the sharp structural asymmetry between the two entities and by partial superficial similarity. Both systems (we treat the pistol shrimp as a mechanical object for the purposes of our study) deliver a powerful expulsion of force – the rock breaker through pressurized percussion impact and the pistol shrimp through a cavitation shockwave generated by a snapping claw.

Each however operates according to distinct principles. The pistol shrimp leverages power amplification – it accumulates and stores energy slowly, then releases it instantaneously. The resulting explosive water jet (accelerated to 25 m/s) forms a vaporized bubble which collapses and lethally stuns the shrimp’s prey. The rock breaker on the other hand delivers high force and continuous power through pressurized liquid. It lacks however the shrimp’s energy accumulation mechanism and is ill-suited to explosive bursts of energy.

This adversarial pairing, combined with strict provenance and physics constraints, created the conditions for our experimental framework.

2.2 Experimental Framework

The source specifications for both systems were developed according to a three-stage LLM-facilitated workflow: identifying authoritative sources, extracting technical parameters with accompanying citations, and structuring data according to the Engineering Decomposition Framework (EDF). This framework decomposes mechanical systems across six hierarchical levels (Table 1) progressing from high-level governing concepts (L0) to precise quantitative parameters (L5).

Table 1: EDF Abstraction Hierarchy

| Level | Focus | Example |
| --- | --- | --- |
| L0 | Governing concept and operating principle | Hydraulic-powered cavitation mechanism for rock breaking |
| L1 | Energy flow sequence from input to output | Hydraulic input → elastic storage → latch release → fluid acceleration → cavitation |
| L2 | Domain primitives from closed lists | A2+A3 (hydraulic+elastic), B2 (latch), C1 (linear), D2 (fluid), E2 (sealed), F2 (electronic) |
| L3 | Specific mechanisms implementing primitives | Latch-release valve, pressure accumulator, linear guide rail |
| L4 | Component parts list and materials | Steel housing, piston assembly, hydraulic lines, tendon cables |
| L5 | Quantitative parameters with units | 210 bar pressure, 750 J stored energy, 25 m/s jet velocity, 40 ms cycle time |

Table 2: Functional Domains

| Domain | Code | Primitive |
| --- | --- | --- |
| A: Energy Source | A1 / A2 / A3 / A4 | Metabolic · Hydraulic · Pneumatic/Elastic · Electrical |
| B: Energy Release | B1 / B2 / B3 | Continuous Cycle · Instantaneous Latch · Direct Drive |
| C: Kinematic Constraint | C1 / C2 / C3 / C4 | Linear Guide · Revolute Joint · Cam/Follower · Linkage |
| D: Working Interface | D1 / D2 / D3 | Percussion Impact · Fluid Acceleration · Grasping/Friction |
| E: Sealing/Containment | E1 / E2 / E3 | Static Seal · Dynamic Seal · Ambient/Open |
| F: Control Interface | F1 / F2 / F3 / F4 | Mechanical Pilot · Electronic Discrete · Neural/Biological · Programmatic |

At L2, designs must select primitives from six functional domains: Energy Source (A), Energy Release (B), Kinematic Constraint (C), Force Delivery (D), Sealing/Containment (E), and Control Interface (F). Domain codes are drawn from a closed list (e.g., A2=hydraulic, B2=instantaneous latch, D2=fluid acceleration) to enforce methodological consistency and prevent invalid combinations.

Every element in a specification carries a provenance tag indicating its origin (Table 3). The synthesis agent operates under defined constraints - it must ground its outputs in the source specifications. Novel elements are permitted only as a last resort and are subject to explicit justification. This prevents unverified hallucination of features derived from the model's general knowledge of pistol shrimp and hydraulic systems.

Table 3: Provenance Tag System

| Tag | Meaning | Usage Priority | Justification Required |
| --- | --- | --- | --- |
| [FROM-A] | Direct transfer from Spec A (Pistol Shrimp) | Highest | No |
| [FROM-B] | Direct transfer from Spec B (Rock Breaker) | Highest | No |
| [ADAPT-A] | Modified from Spec A with stated changes | Medium | Modification must be explicit |
| [ADAPT-B] | Modified from Spec B with stated changes | Medium | Modification must be explicit |
| [SYNTH] | Novel element from neither source | Lowest (last resort) | Must explain why sources inadequate |

Provenance claims undergo active verification through a "mini-RAG" system. The structural validator maintains access to the source specifications as YAML files and searches these documents when encountering provenance tags. A claim of "[FROM-A] plunger diameter 25mm" triggers a search of Spec A for this technical specification to ensure strict grounding in the curated specifications. Designs exceeding 30% SYNTH content are flagged for excessive invention rather than synthesis.

2.3 Multi-Agent Architecture

The system architecture employs three LLM-based agents - a synthesis agent, a physics validator agent, and a structural validator agent - coordinated by Python orchestration code. This hybrid architecture combines LLM reasoning with control flow logic to ensure consistent validation and methodological rigor.

Figure 1: System Architecture and Information Flow

Synthesis Agent

The synthesis agent operates under defined constraints enforced through prompt instructions. It is instructed to prioritize information from Specification A (pistol shrimp) and Specification B (rock breaker) and to resort to novel synthesis only when neither source can serve the required function. It must tag every element with provenance codes indicating its origin. The agent receives validated output from previous synthesis stages as well as feedback from failed validation attempts. It operates without knowledge of retry limits or orchestration logic.

The model temperature was set to 0.4 to enable creative exploration of the design space while maintaining grounding in source material. The agent is required to "show its work" for quantitative claims at L5 and to provide explicit calculations that validators can verify.

Physics Validator

The physics validator uses an independent calculation protocol. Rather than simply verifying that calculations are shown, the validator is instructed to extract parameter values from the synthesis specification, perform arithmetic independently, and compare its results to the agent's claims. The validator operates at a temperature of 0.1 to promote consistent outputs across executions. It enforces thresholds defined in YAML configuration files which serve as a source-grounded baseline.

Structural Validator

The structural validator works to prevent source hallucination through active document verification. It accesses Source Specifications A and B (YAML files) and searches these documents to verify provenance claims. If a discrepancy is found, a failure outcome is produced with feedback specifying the divergence. This "mini-RAG" system enforces grounding in source-based specifications and prevents reliance on the agent's general knowledge about pistol shrimp or hydraulic systems.

The validator also enforces parsimony by requiring explicit justification for every [SYNTH] element. The agent must explain why neither source specification can serve the required function. Vague justifications such as "for better performance" or "to optimize design" trigger immediate failure. Valid justifications must specifically explain why each source is inadequate in supporting the proposed function. The model temperature was set to 0.1 to ensure consistent enforcement of structural constraints across validation runs.

Orchestration and Control Flow

Python orchestration code coordinates agent interactions and manages synthesis progression. The output of the synthesis agent routes to both validators in parallel for independent evaluation. The verdict aggregator combines their outputs using fixed rules: any constraint failure yields FAIL, flags without failures yield PASS_WITH_FLAGS, and outputs without any issues yield PASS. The orchestrator has no LLM component. All decision logic is deterministic code to ensure consistent behavior.

2.4 Constraint System

The physics and structural validation agents together employ 22 constraints applied across two tiers. Universal physics constraints (physics.yaml) apply to all designs regardless of the mechanisms chosen, whereas mechanism-specific prerequisites (mechanism_prerequisites.yaml) are invoked only when specific mechanisms require additional validation against source system parameters.

Table 4: Physics Constraints

| ID | Name | Formula/Check | Applies | Example Violation |
| --- | --- | --- | --- | --- |
| P1 | Energy Conservation | E_available ≥ 0.5mv² | L5 | Design 5: 400 J cable capacity, system needs 750 J |
| P2 | Temporal Feasibility | T_cycle ≥ T_charge + T_fire + T_reset | L5 | Claimed 40 ms cycle, phases sum to 48 ms |
| P3 | Mechanism Prerequisites | Alignment with requirements in mechanism_prerequisites.yaml | L5 | Cavitation at 18 m/s (threshold: ≥25 m/s) |
| P4 | Dimensional Consistency | Units consistent across calculations | L5 | Mixing meters with millimeters in force calculation |
| P5 | Power Adequacy | P_source ≥ E_cycle × f_cycle | L5 | Architecture 2: 21 kW input, claims 80 kW sustained output |
| P6 | No Magic Physics | Obeys known physics laws | All | Perpetual motion, energy from nothing |
| P7 | Energy Continuity | Output connects to next input | L1 | Gap in energy flow chain |
| P8 | Cycle Closure | Returns to initial state | L1 | One-shot sequence, can't repeat |
| P9 | Rate Limiter Identified | Slowest phase identified (flag) | L1 | Bottleneck not specified |
| P10 | Kinematic Continuity | Force paths physically connected | L3 | Force "teleportation" without transmission path |
| P11 | Material Reality | Real industrial materials only | L4 | Fictional "unobtanium alloy" |
| P12 | Efficiency Plausibility | Flag if claimed efficiency >95% | L5 | 97% efficiency claim (flag for review) |

Table 5: Structural Constraints

| ID | Name | Check | Focus |
| --- | --- | --- | --- |
| S1 | Closed-List Compliance | Valid SEDF codes, no prohibited combinations | B1+B2 mutually exclusive |
| S2 | Provenance Accuracy | Mini-RAG verification against source specs | [FROM-A] claims must exist in Spec A |
| S3 | Domain Coverage | All six domains (A-F) addressed | Complete functional specification |
| S4 | Vertical Traceability Down | Elements at N trace to parent at N-1 | No orphan elements |
| S5 | Vertical Traceability Up | Elements at N-1 realized by children at N | No dangling requirements |
| S6 | Primitive Coverage | L2 primitives map to ≥1 L3 mechanism | Complete implementation |
| S7 | Mechanism Coverage | L3 mechanisms decompose to L4 components | Complete parts list |
| S8 | Parameter Completeness | Key parameters specified (flag) | Encourage completeness |
| S9 | SYNTH Justification Required | [SYNTH] elements need necessity explanation | Anti-gaming enforcement |
| T1 | Parsimony Compliance | [SYNTH] usage ≤30% of elements (flag) | Statistical source utilization check |

Structural constraints enforce EDF methodology compliance and prevent source hallucination. S2 performs the critical mini-RAG verification, while S9 serves as the primary source grounding mechanism by requiring high-quality justifications for novel synthesis. T1 monitors source composition by flagging designs that rely excessively on invention rather than source-based synthesis.

Table 6: Constraint Activation by Level

| Level | Active Physics Constraints | Active Structural Constraints |
| --- | --- | --- |
| L0 | P6 | S2, S9, T1 |
| L1 | P6, P7, P8, P9 | S2, S4, S5, S9, T1 |
| L2 | P6 | S1, S2, S3, S4, S5, S9, T1 |
| L3 | P6, P10 | S2, S4, S5, S6, S9, T1 |
| L4 | P6, P11 | S2, S4, S5, S7, S9, T1 |
| L5 | P1-P6, P12 | S2, S4, S5, S8, S9, T1 |

This staged constraint application ensures conceptual validity is established early in the pipeline (L0-L2) before expending computational effort to determine quantitative specifications (L5). Quantitative constraints requiring arithmetic verification (P1, P2, P3, P5) are invoked only at L5 where numerical parameters exist.

2.5 Iterative Descent and Validation Protocol

The synthesis operates sequentially with validation at each level of the framework. After the synthesis agent generates content for level N, the orchestrator routes its output to both validators in parallel for evaluation. Each validator returns a structured JSON response with the constraint results. The verdict aggregator combines these results using defined rules: any constraint failure from either validator yields FAIL, flags without failures yield PASS_WITH_FLAGS, and no violations yields PASS.

Each level permits three validation attempts before triggering backtracking. Failed attempts receive feedback from the validators specifying which constraints were violated, the threshold values versus actual values for quantitative constraints, and specific remediation options.

If all three attempts at level N fail validation, the orchestrator abandons the current synthesis trajectory and restarts synthesis from L0. The system saves the failed synthesis trajectory’s completed levels as a partial design attempt and then extracts a failure signature comprising constraint violation counts and the L2 domain primitive combination (e.g., "A2+A3|B2|C1|D1+D2|E2|F2"). This signature is injected as a negative constraint into the new L0 context instructing the synthesis agent to explore a fundamentally different design approach and avoid re-exploring primitive combinations that led to irreconcilable conflicts.

RESULTS SECTION

Results

The synthesis framework generated seven discrete designs across its full execution. No design passed L5 parameter specifications. Three designs (Designs 2, 4, and 6) were terminated before reaching L5. The remaining four (Designs 1, 3, 5, and 7) reached L5 but failed to pass validation within the permitted retry attempts. Design 7 came closest to completion and accumulated only 8 total violations across its entire run. Design 1 accumulated the most at 95. Table 7 provides an overview of all seven designs.

Table 7: Architecture Overview

| Design | Fingerprint | Completed Levels | Total Violations | Total Tags | Status |
| --- | --- | --- | --- | --- | --- |
| 1 | A2+A3 / B2¦B1 / C1¦C2+C4 / D2¦D1 / E2+E3 / F2 | L0–L4 | 95 | 146 | Failed L5 (9 attempts) |
| 2 | A2+A3 / B2 / C1 / D2 / E2 / F2 | L0–L3 | 17 | 56 | Terminated at L4 |
| 3 | A2+A3 / B2 / C1 / D1 / E2 / F2 | L0–L4 | 80 | 216 | Failed L5 (9 attempts) |
| 4 | A2+A3 / B2 / C1+C4 / D2 / E2 / F2 | L0–L2 | 30 | 79 | Failed at L3 |
| 5 | A2+A3 / B2 / C1 / D1+D2 / E2 / F2 | L0–L4 | 91 | 153 | Failed L5 (9 attempts) |
| 6 | A2+A3 / B2 / C1 / D2 / E2 / F2 | L0–L3 | 9 | 115 | Terminated at L4 |
| 7 | A2+A3 / B2 / C1+C2 / D2 / E2 / F2 | L0–L4 | 8 | 144 | Final state |

Across their domain fingerprints, all seven designs shared the following selections: A2+A3 (hydraulic + elastic energy source), B2 (instantaneous latch release), E2 (sealed containment), and F2 (electronic control). The designs diverged primarily in their kinematic constraint (C) and force delivery (D) selections. Design 1 was the only architecture to use mode-dependent primitives denoted by the "|" operator. It was also the only design to employ E2+E3 hybrid sealing.

Table 8: Constraint Violations by Type

| Design | P1 | P2 | P3 | P4 | P5 | P6 | P10 | S2 | S7 | S9 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 27 | 13 | 9 | 2 | 8 | 1 | — | 22 | — | 13 | 95 |
| 2 | — | — | — | — | — | — | — | 9 | 2 | 6 | 17 |
| 3 | 8 | — | 15 | — | 3 | 2 | — | 38 | — | 14 | 80 |
| 4 | — | — | 3 | — | — | — | 2 | 13 | — | 12 | 30 |
| 5 | 16 | 15 | 8 | — | 9 | 2 | — | 25 | — | 16 | 91 |
| 6 | — | — | 2 | — | — | 1 | — | 4 | — | 2 | 9 |
| 7 | 1 | — | — | — | 1 | — | — | 4 | — | 2 | 8 |

Energy conservation (P1) was the most frequent physics violation. Design 1 recorded 27 P1 violations - the highest single-constraint count of any design. Provenance accuracy (S2) was the most frequent structural violation with Design 3 recording 38 - the highest S2 count across all designs. SYNTH justification (S9) failures were distributed broadly with Designs 5 and 3 recording the highest counts (16 and 14 respectively). Temporal feasibility (P2) violations appeared only in designs attempting dual force delivery: Designs 1 (13) and 5 (15). Kinematic continuity (P10) violations were unique to Design 4.

Table 9: Constraint Violations by Level

| Design | L0 | L1 | L2 | L3 | L4 | L5 | Total |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 7 | 88 | 95 |
| 2 | 0 | 4 | 0 | 1 | 12 | — | 17 |
| 3 | 0 | 5 | 0 | 0 | 10 | 65 | 80 |
| 4 | 0 | 0 | 4 | 26 | — | — | 30 |
| 5 | 0 | 0 | 0 | 0 | 1 | 90 | 91 |
| 6 | 0 | 1 | 0 | 3 | 5 | — | 9 |
| 7 | 1 | 0 | 0 | 0 | 0 | 7 | 8 |

Violations were overwhelmingly concentrated at L5 for the four designs that reached that level. Design 5 accumulated 90 of its 91 total violations at L5 after passing L0 through L3 cleanly. Design 1 followed a similar pattern with 88 of 95 violations at L5. Design 4 presented the sharpest contrast with 26 of its 30 violations occurring at L3 where mechanism specification proved irreconcilable with its C1+C4 kinematic selection. For designs 2 and 6 - both terminated at L4 - violations were distributed more gradually across L1, L3, and L4.

Table 10: Provenance Distribution

| Design | FROM-A | FROM-B | ADAPT-A | ADAPT-B | SYNTH | Total | SYNTH % |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 8 (5.5%) | 46 (31.5%) | 24 (16.4%) | 3 (2.1%) | 65 (44.5%) | 146 | 44.5% |
| 2 | 11 (19.6%) | 11 (19.6%) | 12 (21.4%) | 10 (17.9%) | 12 (21.4%) | 56 | 21.4% |
| 3 | 18 (8.3%) | 134 (62.0%) | 38 (17.6%) | 5 (2.3%) | 21 (9.7%) | 216 | 9.7% |
| 4 | 20 (25.3%) | 22 (27.8%) | 15 (19.0%) | 6 (7.6%) | 16 (20.3%) | 79 | 20.3% |
| 5 | 30 (19.6%) | 44 (28.8%) | 25 (16.3%) | 20 (13.1%) | 34 (22.2%) | 153 | 22.2% |
| 6 | 26 (22.6%) | 26 (22.6%) | 29 (25.2%) | 5 (4.3%) | 29 (25.2%) | 115 | 25.2% |
| 7 | 22 (15.3%) | 40 (27.8%) | 36 (25.0%) | 13 (9.0%) | 33 (22.9%) | 144 | 22.9% |

Design 3 drew most heavily from a single source with 62.0% of its tags originating directly from the rock breaker specification - consistent with its decision to drop cavitation and commit to percussion. Design 2 had the most balanced source distribution with near-equal representation across all five tag categories. Design 1 had the highest SYNTH element proportion at 44.5% which was well above the 30% parsimony threshold. This was driven by the mode-switching components required by its dual-mode design. Design 7 maintained a moderate SYNTH rate of 22.9% while achieving the strongest overall progression.

Design 1: Dual-Mode Material Processing System

Design 1 attempted the most ambitious synthesis: a system capable of switching between cavitation and percussion. Rather than combining both force delivery mechanisms simultaneously, it treated them as discrete operational modes with mode-dependent domain primitives. This required hybrid sealing (E2+E3) to support operation in both fluid-coupled and dry environments. Design 1 was the only design to employ mode-dependent notation and the only design with hybrid sealing.

The design passed L0 through L3 cleanly before accumulating seven violations at L4 as mode-switching hardware introduced elements not traceable to either source. L5 was deeply problematic producing 88 violations across nine failed attempts. Energy conservation violations (P1) dominated with 27 occurrences - the dual-mode architecture could not satisfy energy budgets for both operational paths simultaneously. The provenance profile was heavily skewed toward novel synthesis with SYNTH at 44.5%. This was the highest of any design and well above the 30% parsimony threshold.

Design 2: Pure Cavitation (Minimalist)

Design 2 dropped percussion entirely and committed to pure cavitation. Its fingerprint was the simplest of any design with a single primitive per domain and no combinations or alternatives. It shares its fingerprint with Design 6. Both attempted to scale the pistol shrimp's cavitation mechanism to industrial dimensions and both were terminated at L4.

Unlike most designs, Design 2 showed violations early with failures appearing at L1, L3, and L4 rather than clustering at a single level. It was the only design to incur S7 (domain compatibility) violations. Its provenance distribution was the most balanced of any design with near-equal representation across all five tag categories and a SYNTH rate of 21.4%.

Design 3: Hydraulically-Charged Elastic Impact (Pure Percussion)

Design 3 took the opposite approach relative to Design 2. It dropped cavitation entirely and committed to pure percussion (D1). It applied the pistol shrimp's power amplification principle - elastic storage with an instantaneous latch release (B2) - to the rock breaker's percussive force delivery mode. Its L0 reasoning explicitly referenced prior design failures. This B2–D1 pairing was unique among all seven designs.

Provenance issues emerged immediately at L1 as the cross-domain abstraction and transfer of principle proved too difficult to ground. L4 accumulated 10 S2 violations despite the highest tag count of any design at that level (96 tags). L5 produced 65 violations across nine attempts with mechanism prerequisites (P3) as the leading physics failure at 15 occurrences. The provenance profile was dominated by FROM-B at 62.0% which was by far the highest single-source reliance across all designs. This was consistent with its commitment to the rock breaker's percussion mode. SYNTH was the lowest of any design at 9.7%.

Design 4: Cavitation with Linkage Kinematics

Design 4 added linkage mechanics (C4) to the cavitation-focused approach selecting C1+C4 to leverage the pistol shrimp's linkage-based power amplification alongside linear guidance. It was the shortest-lived design and completed only L0 through L2 before failing at L3.

Performance at L3 was deeply problematic: 26 of its 30 total violations occurred at L3 across three failed attempts. The C1+C4 combination created motion discontinuities that the validators rejected and Design 4 was the only design to incur P10 (kinematic continuity) violations. SYNTH justification failures (S9) dominated at 12 occurrences as the validators rejected the invented linkage mechanisms as unjustified. The provenance distribution was relatively balanced with the highest FROM-A count at L0 (6 tags) of any design indicating strong initial grounding in the pistol shrimp specification.

Design 5: Cavitation-Enhanced Percussion

Design 5 combined both force delivery modes (D1+D2) with simple linear kinematics (C1) and attempted to use cavitation and percussion together rather than as discrete alternatives. This was a simpler implementation of the dual-mode concept than Design 1 and without mode switching.

The design passed L0 through L3 without a single violation before producing 90 of its 91 total violations at L5. No other design exhibited such an abrupt collapse. Unlike other high-violation designs, failures were distributed broadly across constraint types with no single dominant mode. Its temporal feasibility (P2) count of 15 was the highest of any design and the combined force delivery commitment created timing conflicts even without explicit mode switching. Design 5 had the highest ADAPT-B count and percentage (13.1%) of any design which reflected substantial adaptation of rock breaker components.

Design 6: Pure Cavitation (Second Attempt)

Design 6 shared Design 2's fingerprint. It represented a second attempt at the minimalist pure cavitation approach. It completed L0 through L3 and was terminated at L4. Its total of 9 violations was the second lowest of any design.

Despite identical domain primitive selections, the two cavitation designs diverged in specification and failure profile. Design 6 had higher total tags (115 vs. 56) and a higher SYNTH proportion (25.2% vs. 21.4%). At L3, SYNTH dramatically rose to 19 tags (37% of the level) - the highest L3 SYNTH count of any architecture. Design 2 struggled with distributed violations from L1 onward, whereas Design 6 progressed more cleanly before encountering difficulties at L3.

Design 7: Cavitation with Revolute Joint Kinematics

Design 7 selected C1+C2 (linear guide + revolute joint) for cavitation-only force delivery (D2) using the revolute joint's rotational motion to generate cavitation. It completed L0 through L4 and reached L5 with only 8 total violations - far fewer than any other design.

A single S2 violation appeared at L0 followed by clean passage through L1 to L4. L5 produced just 7 violations. Design 7 was the only architecture with zero P2 (temporal feasibility) and zero P3 (mechanism prerequisites) violations, and its physics failures were limited to two instances across the entire run. At L2, SYNTH was minimal (1 of 17 tags) - the lowest of any design - indicating near-complete source grounding at the domain primitive level. At L1, it had the highest FROM-A count (11 tags) of any design reflecting strong direct transfer of pistol shrimp mechanisms into the energy flow specification.

DISCUSSION AND ANALYSIS

Our system explored discrete designs and surfaced a range of engineering principles and technical patterns. But the experimental scope was deliberately limited - we designed a hierarchical constrained synthesis with an adversarial pairing featuring irreconcilable systems. Consequently, no significant engineering outcome was expected to emerge. The primary value of these results however is in what they reveal about the language model’s reasoning capabilities and the feasibility of using AI to orchestrate engineering design.

We examine specific design decisions below - both failures and successes - before turning to broader questions about AI's capacity for creative engineering and scientific discovery.

Spurious Reasoning

One recurring and conspicuous deficiency in our results was the language model’s tendency to generate plausible-sounding rationales disconnected from engineering rigor and reality. For instance, the L0 reasoning of Design 1 states that a dual-mode approach allows the system to “leverage the shrimp’s precision and energy efficiency for delicate operations”. This led it to enlist the shrimp’s “high-velocity fluid mechanism” for “precision applications requiring cavitation effects”.

This however betrays a lack of semantic understanding of these concepts. Cavitation is a fundamentally violent and chaotic phenomenon. The collapse of the pistol shrimp's cavitation bubble generates shockwaves and temperatures briefly exceeding 4700°C. Arguing that this would benefit the synthesized object’s ability to undertake “precision” or “delicate” operations is clearly implausible and erroneous.

This phenomenon is likely an artifact of the underlying mechanics of language models. As statistical prediction engines, LLMs are prone to spurious associations. It appears that the language model is conflating the small-scale properties of the shrimp’s cavitation mechanism with ideas of precision.

This points to a limitation of our experimental design. Namely, while we defined constraints to ensure provenance tracking, EDF compliance, and consistency with physics rules, we did not apply a control to validate semantic understanding.

Given the demonstrable value of self-critique and refinement in enhancing language model performance, our work would have benefited from introduction of a “critic” or “red team” agent. Such an agent would have been instructed to critically review the outputs of the synthesis at earlier, more conceptual stages.  The critical feedback provided would have helped mitigate erroneous reasoning during idea incubation and would have anchored the generated ideas in a more rigorous foundation.

Admittedly, this flawed line of reasoning did not exert any real influence on further synthesis progression - the design was ultimately undermined by its difficulty satisfying structural validation constraints. Regardless, we must be vigilant of such missteps. Early divergences, errors, and misinterpretations, however small, can propagate through the pipeline and expand in magnitude as they amplify across decision nodes.

Flawed Causal Inference and Long-Term Dependencies

While our validation constraints helped guard against outright errors, the model still made subtle reasoning mistakes related to multi-step causal inference. We examine three prominent examples:

- Design 5: Component undersized for specification. At L1, the system committed to storing 750 J of elastic energy per cycle. At L4, when specifying the component storing this energy, the agent described a "12mm diameter high-strength steel cable with 100mm active length to achieve 400 J storage capacity." The component's maximum capacity is 400 J, but the system requires 750 J. The system is 350 J short of its own specification.

- Design 2: Claimed performance exceeds available power. At L0, the system promises "1-10 kJ per pulse." L1 repeats this claim while also specifying a 21 kW hydraulic input and 32-121 ms cycle times. But energy per pulse depends on the amount of time the system has to accumulate energy from a fixed-power source before firing. At 21 kW, accumulating 1 kJ requires 48 ms and accumulating 10 kJ requires 476 ms. The specified cycle time range of 32-121 ms only allows 672-2,541 J per pulse. The 10 kJ upper bound is impossible at any of the specified cycle rates as it would require nearly four times the maximum cycle time. Even the 1 kJ lower bound is only achievable at cycle times above 48 ms which excludes the faster end of the claimed operating range. The agent could not effectively navigate the trade-off between high energy per pulse and high cycling frequency.

- Design 6: Biological parameter copied without scaling. At L1, the latch holding force is specified as 50 N - the exact value from the pistol shrimp's biological specification. At L3, the elastic storage system is described as "25mm diameter" tendons charged by "150 bar" hydraulic pressure. Hydraulic pressure at that level on any reasonable piston area produces forces in the kilonewtons. Design 3, facing the same problem, correctly scaled its latch to hold 10,500 N. Design 6 however did not make the corresponding adjustment. A 50 N latch cannot restrain kilonewton-scale charging forces – the latch would be released the instant hydraulic pressure was applied. The agent transferred a source parameter that required scaling but failed to perform the scaling.

These three failures share a common deficit - the model reasons locally rather than globally. Design 5 specified a component without checking it against an energy commitment made two levels earlier. Design 2 selected three independently reasonable parameters (energy per pulse, cycle time, and power input) without recognizing their joint incompatibility. Design 6 transferred a parameter without performing the appropriate scaling. In each case, the model produced specifications that were locally but not globally coherent.

Validation and constraints can mitigate these, but they cannot permanently overcome the structural limitations of model reasoning. Language models have demonstrated considerable fluency on well-defined problems, but this fluency degrades as the model must maintain consistency across multiple levels of transformation. This challenge compounds with system complexity. Heuristics like incorporating explicit verification into prompt instructions become harder to enforce as the number of long-term dependencies grows.

AI’s Capacity for Creative Design

The model's reasoning however was not always disjointed. Four designs exhibited moments of genuine creative problem-solving as it attempted its synthesis.

Design 1 – Dual-Mode

In Design 1, the model envisaged a dual-mode system sequentially switching between cavitation and percussion. It saw this as synergistic and defended it with a sound rationale: percussion breaks up hard rock first, then cavitation attacks the fractures created by the initial impact. This represents an effort at genuine synthesis and has the trappings of an emergent property - a capability that exists in neither source system alone but emerges from their combination.

More striking was a form of structural creativity not seen in any of the other designs. The prompt instructions indicated that functional domains could be combined (barring B1+B2) but provided no explicit conception of alternating or switching between modes. Design 1 independently invented a new notation - the "|" character - to denote a dual-mode system signifying sequential operation in "either mode." The model effectively extended the framework's expressiveness to accommodate its proposed design. This came however at a considerable cost as 44% of all provenance tags associated with Design 1 were tagged SYNTH.

Design 3 – Concept Abstraction

Design 3 performed a subtler feat of creative reasoning: it extracted a principle from one source while discarding the mechanism that embodied it. This allowed it to achieve a more robust synthesis without incurring the constraint violations other designs contended with. Understanding how it did this calls for some explanation of the energy systems of the two sources.

The pistol shrimp’s cavitation mechanism has three components: slow energy accumulation, instantaneous release, and forced delivery via cavitation. The rock breaker’s striking mechanism however is governed by a different dynamic – hydraulic power drives a continuous cycle of pressurized fluid which delivers percussion impact.

Design 3 abstracted the principle of instantaneous release (B2) and routed it to the rock breaker’s delivery mode (D1) forgoing the use of cavitation and fluid acceleration (D2). This B2-D1 pairing was unique among all the designs. It allowed Design 3 to escape a profound challenge - scaling fluid acceleration to industrial dimensions. At an industrial scale, accelerating fluid volume to extreme velocities (e.g. 25+m/s) has a disproportionate energy cost and this repeatedly created physics constraint violations. Design 3 seized the power amplification benefit without having to solve cavitation scaling.

Although design 3 had a relatively low number of physics violations, it had the highest number of provenance violations due to this novel combination. The structural validator coupled fluid acceleration with cavitation. In discarding cavitation and associating fluid acceleration with percussion instead, Design 3 was repeatedly penalized by provenance constraints.

Design 4 – Parallel Charging

Design 4 creatively addressed a core timing constraint. The pistol shrimp’s latch-release mechanism requires 320ms to recharge between cycles. The rock breaker however operates at 25 Hz which translates to strikes every 40ms. A single shrimp style latch-release mechanism cannot recharge fast enough to sustain this rate. Design 4 proposed using eight energy accumulators in parallel and in so doing independently arrived at a time-tested engineering strategy seen in systems like multi-cylinder engines.

Design 4 however suffered from a deficiency not exhibited by any other design – the quality of its SYNTH justifications rapidly deteriorated. The framework requires that every SYNTH component be accompanied by an explanation of why it is necessary and why neither source can serve the required function. While other designs provided robust justifications, Design 4's justifications declined substantially in quality and specificity. This is presumably why it was terminated earliest and failed to pass L3.

Why did performance deteriorate mid-execution? A definitive explanation is impossible given that LLMs are stochastic and their outputs can fluctuate unpredictably. Our decision to use a relatively high model temperature (0.4) for the synthesis agent may have exacerbated this. Other contributing factors could include context window pressure or the difficulty of reasoning coherently about eight parallel accumulators simultaneously.

Creativity and Artificial Intelligence

Assessing and opining on creativity requires some working criterion. Rather than build a philosophically fraught formal definition, we apply a modest standard: a design choice qualifies as creative when it is surprising, impressive, or when it produces emergent capabilities not present in either source system. This can be distilled into an even more straightforward test: if a human engineer had made this decision, would it have elicited a similar reaction?

This rubric however must be placed in context. The rock breaker has been intensively studied, engineered, and refined over decades. Its present form reflects cumulative optimization across performance, cost, energy efficiency, manufacturability, and operational complexity. Our synthesis was therefore not likely to improve on this in any way. Even a coherent hybrid construction was improbable given the selected adversarial pairing with its irreconcilable energy systems.

Yet against this backdrop the designs were not just lackluster write-offs. While they produced no breakthroughs, they demonstrated an ability to effectively explore the constrained design space. Even more striking, they showed genuine synthesis that went beyond uncritical recombination.

AI and the Structure of Engineering Progress

The most ambitious test of creative engineering design must grapple with the most significant driver of engineering progress: groundbreaking breakthroughs. Our system did not produce such breakthroughs. But the question extends beyond our experiment - could an AI system be capable of such innovation? There are reasons to be skeptical.

The history of the engine is instructive. With some simplification, we can view progress in engine design as advancing along two tracks: long stretches of incremental refinement punctuated by moments of radical restructuring and redesign. Consider Watt's introduction of a separate condenser, the double-acting cylinder, or the steam turbine. These were not minor optimizations within existing designs but instead represented fundamental reconsiderations of what an engine could be. The separate condenser required recognizing that heating and cooling the same cylinder was not a problem requiring superior permutations but a structural flaw to be entirely reengineered.

This juxtaposition is of considerable importance when considering AI’s role in technical pursuits. Optimization of constraints - adjusting parameters, substituting materials, exploring combinations - is precisely the kind of guided search that language models excel at. But structural redesign and reconception are fundamentally different activities. They require identifying that a framework's foundational assumptions are themselves the limiting factors and that the search space needs to be reinvented rather than fully explored.

Language models have access to vast and interdisciplinary knowledge, and it may be argued that the raw materials for such insights are to be found in their training data. However, the mere presence of disparate ideas and techniques cannot ensure that they will be joined together in a powerful reconceptualization. For example, a model trained on every engine design before Watt would contain information about heat loss, thermal cycling, and the separation of functions in other mechanical contexts. But would an architecture based on statistical prediction surface such a radical recombination as a candidate solution? This would require overcoming the implicit tendencies in the distribution of the model’s training data to generate statistically likely outputs.

Our results do not provide evidence of a model making such foundational jumps. The model explored the design space systematically and produced genuine moments of creative recombination. But nothing in its outputs suggested the capacity to offer radical restructuring that would overturn orthodoxy and bring new engineering ideas to the fore.

At the same time, the engine's history also reveals where AI's real value may reside. Alongside the radical reconsiderations, engines improved through the steady accumulation of smaller advances: more rigorous thermodynamic analysis, higher-temperature alloys, better lubricants, electronic fuel injection. These incremental, compounding refinements were collectively consequential and enduring. This is the domain most conducive to AI acceleration. At its core, the language model functions as a powerful search engine - one that can systematically explore parameter spaces, test material configurations, and navigate constraint landscapes far more efficiently than manual intervention.

Our results demonstrate this. The system explored seven discrete designs while navigating constraints, decomposing abstract primitives into concrete parts lists, and identifying unproductive domain primitive combinations to avoid in subsequent attempts. This suggests a natural division of labor: AI accelerates design space exploration and constraint navigation while human engineers supply the judgment, semantic understanding, and capacity for reconceptualization.

Strengthening Engineering AI Methodologies

Noting the limitations of language models for engineering design, would more compute translate to innovative breakthroughs? The history of model development provides a negative answer. Progress in machine learning has most often been driven by identifying new and robust inductive biases - the structural assumptions about the nature of the distribution and what learning representations are best suited to capture them.

This follows a well-known pattern. Recognizing that image pixels maintain spatial relationships with translational invariance led to convolutional neural networks. Recognizing that contextual relationships vary in significance across sequential text led to transformers. In each case, capturing the right structural assumptions about the data produced dramatic performance gains.

The transformer architecture has demonstrated strong reasoning fluency, but there are grounds to question whether its inductive biases are sufficient for engineering innovation and scientific discovery. The engine breakthroughs we examined earlier were fundamentally acts of structural redesign. These require discontinuous leaps that had no immediate precursors. An autoregressive model operating on token sequences may be fundamentally limited in approximating this behavior.

What such an architecture would look like remains an open question. It would likely need to encode concepts like invariance, equivalence, and symmetry as representational primitives rather than emergent properties of token prediction. Whether this is possible - and whether it would be sufficient for the kind of structural redesign that drives paradigm-shifting breakthroughs - is a consequential yet unanswered question.

Future Considerations

Our linear pipeline was a deliberate choice. It provided a tractable, principled, and interpretable execution of the AI-orchestrated design process. But engineering design is fundamentally nonlinear. Engineers routinely move across levels of abstraction, refactor structures, and simultaneously balance competing constraints. Our system's unidirectional progression restricted this kind of fluid and free-form exploration.

A more flexible architecture would likely move toward multi-agent frameworks with a different allocation of responsibilities. Consider a custodian agent charged with maintaining the coherence of the base design, an innovator agent tasked with evolving components toward the synthesis target, and a mediating agent that adjudicates conflicts between them. This reflects a top-down approach to synthesis in contrast to our bottom-up construction.

Our own results point to the need for such flexibility. Design 3 was penalized for one of the more creative decisions in the experiment (selective abstraction of an energy mechanism) because the validator could only enforce rigid provenance rules. Design 1's semantic errors went unchallenged because no agent was charged with evaluating conceptual coherence. A system where agents can negotiate, challenge, and mediate rather than simply output a pass or fail outcome would better accommodate the kind of robust synthesis we desire.

Another option for enhancement is through process parallelization. An updated system could run concurrent pipelines with different constraint hierarchies prioritizing cost, coherence, or manufacturability.

Whether either approach would yield stronger results remains to be tested, but the architectural limitations our system exposed suggest clear directions for further work.

Limitations

This primary limitations of this paper stem from its experimental scope and evaluation methodology. The adversarial pairing of a pistol shrimp and rock breaker was intentionally designed as an extreme stress test of AI synthesis capabilities. It is possible that the physics of these two systems are fundamentally irreconcilable and the task is intractable. The deep incompatibility of this pairing may preclude drawing insights about AI performance on routine engineering tasks where system compatibility is higher.

As a single case study, generalizability is limited. Results may not extend to other biomimetic pairings, non-adversarial synthesis tasks, or engineering domains beyond mechanical systems. The EDF framework and constraint architecture are specific to this experimental design.

Several evaluation constraints affect interpretation. No human engineers attempted the same synthesis task. This precluded baseline comparison of AI performance relative to expert capabilities. We employed no quantitative metrics for creativity or synthesis quality and instead relied on subjective assessments of creativity based on researcher judgment. Designs were neither physically prototyped nor validated by domain experts and their practical feasibility is unverified.

Methodologically, the system lacked access to external computational tools (e.g., CAD software, physics engines) that might have mitigated the quantitative reasoning failures observed at L5. Additionally, constraint coverage was deliberately incomplete. We validated physics compliance and provenance accuracy but omitted real-world engineering factors including cost, manufacturability, safety, maintainability, and operational complexity.

The study examined a single model architecture (Claude Sonnet 4) with defined temperature settings (0.4 for synthesis, 0.1 for validation). Different model families, prompting strategies, temperature configurations, or computational budgets might yield substantially different outcomes. The token budget limit of 5 million may have prematurely terminated promising architectural explorations.

Despite these limitations, the framework's ability to generate interpretable failure modes and surface identifiable moments of creative reasoning suggests that constrained synthesis pipelines may be a productive framework for studying AI's engineering capabilities.

CONCLUSION

We designed a hierarchical, constrained multi-agent synthesis framework to test whether a language model could synthesize a pistol shrimp and hydraulic rock breaker into a coherent hybrid mechanical construction. The deliberate adversarial pairing ensured failure but created conditions for studying how language models reason under constraints - where they degrade, where they show evidence of coherence and novelty, and what these patterns reveal about the feasibility of AI-orchestrated engineering design.

The system produced seven discrete designs of varying technical complexity. We identified design choices reflecting genuine creative problem-solving – energy mechanism abstraction across domains, spontaneous invention of notation, independent convergence on established engineering strategies. But the system also exposed persistent weaknesses: spurious rationalizations masking semantic misunderstanding, difficulty sustaining multi-step inference, and failure to maintain consistency across long-range dependencies. Overall, however, the system demonstrated competence in exploring the design space and surfacing novel configurations.

Language models are well-suited to optimizing parameters and navigating constraints and this positions them well for accelerating incremental engineering tasks. But revolutionary advances require radical restructuring - discontinuous leaps with no immediate precursors. An autoregressive model operating on token sequences may be fundamentally limited in approximating this kind of reasoning. Nothing in our results suggests that current AI architectures can transcend this limitation.

Advancing AI's role in technical disciplines requires both architectural innovation and effective human-AI collaboration. Architecturally, new inductive biases encoding concepts like invariance, symmetry, and structural equivalence would better equip models for the reasoning demanded by engineering and scientific discovery. In terms of collaboration, our results point toward a more targeted division of labor. This calls for AI systems accelerating design space exploration, component selection, and constraint navigation, and human engineers supply the domain judgment, semantic understanding, and capacity for reconceptualization that models cannot yet provide.

DATA AND CODE AVAILABILITY

The complete source specifications (pistol shrimp and hydraulic rock breaker YAML files), constraint configurations (physics.yaml, structural.yaml, mechanism_prerequisites.yaml, sedf_codes.yaml), system prompts for all agents, the full SEDF framework document, and raw JSON outputs for all seven design trajectories are available at https://github.com/XXX/XXX.