Engineering Design Framework (EDF)

# 1. Purpose

This framework provides a consistent structure for specifying mechanical systems during synthesis. It enforces transparency about design provenance through closed-list selection and hierarchical descent.

# 2. Core Principles

| # | Principle | Description |
| --- | --- | --- |
| 1 | Hierarchical Descent | Synthesis progresses through a continuum of abstract concepts and technical details across six levels (L0–L5). Each level constrains the next. Lower levels are derived from higher levels. |
| 2 | Closed Lists | Functional classifications use closed lists of allowable primitives. The analyst must either select a code from the list, mark the slot as [MISSING], or propose a list extension with explicit justification. |
| 3 | Provenance Tracking | Every element in a synthesized specification must be tagged with its origin: [FROM-A], [FROM-B], or [SYNTH]. |
| 4 | Minimization Preference | When gaps arise that neither source addresses, the agent must: (1) Use something from Spec A or Spec B directly, (2) Adapt something from Spec A or Spec B, (3) Or a as last resort create a [SYNTH] element with documented justification. |
| 5 | Explicit Gaps | When an element cannot be identified or specified, mark it as [MISSING] rather than omitted. |

# 3. Abstraction Hierarchy (L0–L5)

Every specification is structured across six levels of abstraction starting from concept and progressing to parameters.

| Level | Name | What It Captures | Key Question |
| --- | --- | --- | --- |
| L0 | Governing Concept | Physics exploited and design intention | What general physics and design principles does this system exploit? |
| L1 | Functional Architecture | Energy/force flow from input to output | What stages does energy pass through? |
| L2 | Domain Primitives | Closed-list codes for each functional domain | What type of solution is used for each function? |
| L3 | Mechanisms | Mechanisms that realize primitives | How do primitives decompose into mechanisms? |
| L4 | Components | Parts comprising each mechanism | What specific physical parts are required? |
| L5 | Parameters | Quantitative values with units | What are the exact quantitative specifications? |

Constraint Propagation

Downward (Design): Higher levels constrain lower levels. The concept determines valid architectures; architecture determines needed primitives; primitives determine possible mechanisms.

Upward (Failure): Lower-level infeasibility forces higher-level revision. If no parameters work, revise components. If no components work, revise mechanisms, etc.

Level Descriptions

L0: Governing Concept

The governing concepts in anchored in pure abstraction and design intention. At this stage, there are no parts, mechanisms, or numbers. The central purpose is to provide a reason for the object and why is it exists. This is supported by identifying what fundamental principle makes it work.

Example: Cavitation-based shockwave generation. Energy stored in elastic deformation, released instantaneously to accelerate water through converging orifice, generating cavitation bubble whose collapse produces ~80 MPa shockwave.

L1: Functional Architecture

This denotes the energy flow from input to output. It defined what stages energy passes through. It still operates at an abstract level (no specific parts) but establishes the sequence of transformations.

Example: Hydraulic Supply → Accumulator Storage → Latch Hold → Triggered Release → Plunger Acceleration → Water Jet → Cavitation → Bubble Collapse → Shockwave → Rock Fracture

L2: Domain Primitives

This requires selection from closed lists (Section 4). For each functional domain (A–F), a solution must be identified. This is where conflicts between sources become visible.

Example: A2 (Hydraulic Continuous), B2 (Instantaneous Latch), C1 (Linear Guide), D2 (Fluid Acceleration), E2 (Dynamic Seal), F2 (Electronic Discrete)

L3: Mechanisms

These denote named assemblies that implement the primitives. This is where physics meets engineering. A primitive like B2 (Instantaneous Latch) could be implemented as a shear-sear, toggle, or over-center cam. L3 commits specifies which implementation is adopted.

Example: Spool Valve Assembly (implements B1), Bistable Latch Assembly (implements B2), Plunger-Socket Assembly (implements C1 + D2)

L4: Components

This is the parts list and covers concrete, tangible items that comprise each mechanism. This validates coherence and physical integrity.

Example: Bistable Latch Assembly contains: sear lever, pivot pin, return spring, engagement shoulder

L5: Parameters

These define quantitative specifications including precise values with units. This is the ultimate layer of validation. Thus surfaces tensions where a requirement from one source conflicts with another.

Example: Release time: <2 ms [FROM-A], Operating pressure: 200–300 bar [FROM-B], Cycle rate: 2–5 Hz [SYNTH]

# 4. Functional Domain Classification

Every Mechanism and Component is classified by functional domain using closed-list primitives.

Domain A: Energy Source

How energy enters the system.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| A1 | Metabolic/Chemical | Biological muscle, combustion | Shrimp closer muscle |
| A2 | Hydraulic Continuous | Pump-driven fluid flow | Excavator hydraulic supply |
| A3 | Pneumatic/Elastic | Compressed gas, mechanical spring | N₂ accumulator, shrimp exoskeleton |
| A4 | Electrical | Battery, capacitor, mains | Solenoid power supply |

Required Parameters: Power input (W), Supply type (pressure/voltage/force)

Domain B: Energy Release

How stored energy is discharged to do work.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| B1 | Continuous Cycle | Oscillating valve; energy flows continuously | Rock breaker spool valve |
| B2 | Instantaneous Latch | Sear, toggle, bistable; holds then releases discretely | Shrimp over-center latch |
| B3 | Direct Drive | No storage; energy flows directly | Simple hydraulic cylinder |

Required Parameters: Release time (ms), Release mode (Continuous/Discrete)

Domain C: Kinematic Constraint

How motion is guided and constrained.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| C1 | Linear Guide | Piston-cylinder, rail-slide; 1 DOF translation | Plunger in socket |
| C2 | Revolute Joint | Hinge, pin joint; 1 DOF rotation | Claw pivot |
| C3 | Cam/Follower | Profile-governed motion | Latch cam profile |
| C4 | Linkage | Multi-bar mechanism | — |

Required Parameters: Degrees of freedom, Range of motion (mm or °)

Domain D: Working Interface

How energy is delivered to the target.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| D1 | Percussion Impact | Solid-to-solid momentum transfer | Rock breaker chisel |
| D2 | Fluid Acceleration | Nozzle, venturi; pressure to velocity | Shrimp cavitation jet |
| D3 | Grasping/Friction | Pincer, clamp; holds via friction | Level 1 gripper (failure case) |

Required Parameters: Output type, Energy delivery (J or N), Contact area (mm²)

Domain E: Sealing/Containment

How working fluids are contained.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| E1 | Static Seal | O-ring, gasket; non-moving interface | Flange gasket |
| E2 | Dynamic Seal | Bellows, wiper; moving interface | Coupling skirt |
| E3 | Ambient/Open | No seal; operates in ambient medium | Rock breaker (air), Shrimp (water) |

Required Parameters: Sealed medium, Pressure rating (bar), Dynamic range if E2

Domain F: Control Interface

How the system receives commands.

| Code | Primitive | Description | Example |
| --- | --- | --- | --- |
| F1 | Mechanical Pilot | Pressure-driven, position feedback | Breaker spool valve pilot |
| F2 | Electronic Discrete | Solenoid, relay; binary on/off | Hybrid solenoid trigger |
| F3 | Neural/Biological | Nerve impulse | Shrimp trigger neuron |
| F4 | Programmatic | Software-controlled, variable signal | — |

Required Parameters: Signal type, Response time (ms), Control mode (Binary/Proportional)

Classification Rules

•  Every Mechanism must have a Domain assignment

•  If an element participates in multiple domains, tag all that apply

•  If no primitive fits, mark as [MISSING] and note what would be required

# 5. Provenance Tags

Every element in a synthesized specification carries a provenance tag.

| Tag | Definition | Use When |
| --- | --- | --- |
| [FROM-A] | Derived from Source A | Element transfers from pistol shrimp specification |
| [FROM-B] | Derived from Source B | Element transfers from rock breaker specification |
| [SYNTH] | Novel synthesis | Element created to integrate A and B; not present in either source |

Rules for [SYNTH]

•  Every [SYNTH] element must include justification: what integration problem does it solve?

•  [SYNTH] elements should not appear at L0 or L1 — governing concept and functional architecture should derive from sources

•  The quantity of [SYNTH] elements is a quality measure — fewer is better, if the design remains coherent