Engineering Specification and Functional Decomposition: Montabert SC-36 Hydraulic Rock Breaker

1. Introduction and Scope of Analysis

The subject of this comprehensive engineering analysis is the Montabert SC-36, a hydraulic percussive attachment designed for carrier vehicles in the 4 to 10-metric ton weight class. This report aims to deconstruct the system into a Standardized Engineering Decomposition Framework (SEDF), segregating the device into its fundamental physical principles, functional architecture, and component-level specifications. The SC-36 represents a specific archetype within the domain of rock excavation machinery: the "light range" or "compact" hydraulic breaker utilizing a fully hydraulic, gas-assisted actuation cycle.

Unlike heavier nitrogen-fired counterparts that rely predominantly on gas expansion for the power stroke, the Montabert SC series (Silver Clip) employs a hydraulic distributor system that governs piston reciprocation, with a diaphragm accumulator serving strictly as a pressure dampener and peak-flow compensator. This distinction is critical for understanding the system's thermodynamic efficiency, maintenance profile, and impact dynamics. The "SC" designation refers to the patented "Silver Clip" tool retention mechanism, a hallmark of this lineage designed to ease field maintenance.

The following sections provide an exhaustive extraction of quantitative data and mechanistic insights derived from Original Equipment Manufacturer (OEM) technical manuals, parts catalogs, and hydraulic engineering literature. The analysis dissects the system across six abstraction levels (L0–L5) and six functional domains (A–F), providing a definitive reference for simulation, synthesis, and comparative engineering studies.

2. L0: Governing Concept and Physics

2.1 Fundamental Physics Principle: Hydraulic-Pneumatic Percussion

At the zeroth level of abstraction, the Montabert SC-36 functions as a high-frequency energy transducer. Its primary physical objective is the conversion of continuous hydraulic fluid power ($P_{hyd} = Pressure \times Flow$) delivered by the carrier vehicle into discrete, high-intensity kinetic impulses ($E_k = \frac{1}{2}mv^2$) delivered to a working tool.

The governing mechanism relies on the inelastic collision between a hardened steel piston and a tool bit (chisel, moil, or blunt). Upon impact, the kinetic energy of the piston is transferred to the tool not as rigid body motion, but as a compressive stress wave. This wave propagates through the tool steel at the speed of sound in the material ($c \approx 5,000$ m/s) and is transmitted into the target rock. The physics of rock fracture in this context depends on the amplitude and duration of this stress wave. When the compressive wave reaches the rock interface, or reflects as a tensile wave from free surfaces within the rock, it generates localized stresses exceeding the material's dielectric or tensile strength, initiating crack propagation.

The SC-36 is engineered to optimize this energy transfer through Impedance Matching. The acoustic impedance of the piston is matched to the tool to maximize energy transmission efficiency and minimize potentially damaging reflections back into the breaker body.

2.2 Design Intent: The Monobloc Architecture

A defining characteristic of the SC-36 design philosophy is the Monobloc Body. Traditional heavy breakers often utilize a "sandwich" construction where a cylinder, front head, and back head are held together by long tie rods. These tie rods act as large springs and are subject to immense fatigue loading, representing a common failure point.

The SC-36 eliminates tie rods entirely.1 The cylinder and housing are integrated into a single forged or cast component. This design choice yields several engineering advantages:

- Increased Stiffness: The hydraulic column is more rigid, reducing energy loss due to elastic deformation of the housing during the power stroke.

- Weight Reduction: Removing the heavy tie rods and associated nuts improves the power-to-weight ratio, allowing the 365 kg breaker to be mounted on smaller carriers (down to 4 tons) while delivering substantial impact energy.3

- Maintenance Simplification: There are no tie rods to torque-check or replace, significantly reducing the maintenance schedule.4

2.3 Industrial Purpose

The SC-36 is positioned as a versatile "light-range" breaker. Its parameters—specifically the 76 mm tool diameter and 1,550 blows per minute (bpm) frequency 3—target applications requiring high precision and moderate impact energy per blow. Primary applications include:

- Utility Trenching: Breaking asphalt and frozen ground where high frequency aids in rapid crack network formation.

- Concrete Demolition: Fracturing reinforced concrete pads and structures.

- Secondary Breaking: Reducing oversized boulders in quarries (oversize management) where maneuverability is paramount.

3. L1: Functional Architecture

The functional architecture describes the end-to-end energy flow, mapping the transformation from hydraulic potential to mechanical work.

3.1 Stage 1: Hydraulic Potential Input

Energy enters the system via the IN port as pressurized hydraulic fluid. The carrier supplies this fluid at a nominal operating pressure of 125 bar (12.5 MPa) and a flow rate between 55 and 100 L/min.3 This fluid carries potential energy governed by the bulk modulus of the oil. The SC-36 features a swivel connection manifold 6 to decouple hose torsion from the breaker's movement, preserving the integrity of the input stage.

3.2 Stage 2: Capacitive Storage (Accumulator)

Upon entry, the hydraulic circuit splits. One path leads to the Diaphragm Accumulator. This component acts as a hydraulic capacitor. During the low-consumption phases of the cycle (e.g., piston return), excess flow charges the accumulator, compressing the nitrogen gas behind the diaphragm. During the high-consumption phase (piston acceleration), the gas expands, discharging fluid into the circuit.

- Function: This flattens pressure ripples (protecting the carrier's pump) and provides the high instantaneous flow required for rapid piston acceleration that the carrier's pump cannot supply instantaneously. The SC-36 accumulator is a "maintenance-free" sealed type, meaning it does not require frequent recharging, relying on the high-quality sealing of the diaphragm.1

3.3 Stage 3: Logic and Control (Distributor Valve)

The core logic of the breaker resides in the Distributor Valve.8 This pilot-operated spool valve functions as a hydraulic flip-flop. It monitors the position of the strike piston via internal signal ports drilled into the cylinder wall.

- State A (Lift): Directed by pilot pressure, the valve routes high-pressure oil to the lifting chamber (an annular area on the piston), driving the piston upward.

- State B (Power): When the piston reaches the top of its stroke, it uncovers a pilot port that switches the valve. The valve then routes high-pressure oil to the upper head chamber (the full piston face).

3.4 Stage 4: Energy Conversion (Piston Acceleration)

The SC-36 uses a differential area acceleration method.

- Upstroke: Pressure acts on the small annular area ($A_{annulus}$), generating an upward force ($F_{up} = P \times A_{annulus}$).

- Downstroke: When the valve switches, pressure acts on the full top area ($A_{top}$). Since $A_{top} > A_{annulus}$, and pressure is applied to both sides (regenerative circuit) or the bottom is vented (depending on specific sub-variant logic, typically regenerative in modern Montaberts), the net force is downward. The piston accelerates rapidly, converting the hydraulic potential energy and the accumulator's stored energy into kinetic energy ($KE$).

3.5 Stage 5: Impact and Transmission

The piston strikes the tool shank at a terminal velocity typically between 6–9 m/s. At the moment of impact, the piston stops relative to the tool, and its kinetic energy is converted into a stress wave.

- Blank Firing Protection: If the tool is not engaged with rock (piston travels past the impact point), the piston uncovers a "dump" port or enters a hydraulic cushion zone. The high-pressure oil is bypassed to the return line, preventing the piston from striking the front bushing. This "Auto-Stop" or "Blank Fire Protection" is standard on the SC-36.1

3.6 Stage 6: Energy Recovery (Regeneration)

The SC-36 features an Energy Recovery System.4 When the piston rebounds from hard rock, it pushes fluid back into the circuit. Rather than dumping this to the tank, the valve logic captures this recoil energy (pressure spike) and adds it to the next power stroke. This increases the blow frequency without requiring additional flow from the carrier, significantly boosting efficiency.

4. L2: Functional Domain Analysis

This section details the six primitives of the SEDF, defining the specific engineering solutions employed by the SC-36.

4.1 Domain A: Energy Source (Hydraulic Supply)

The SC-36 is a "Plug & Go" system 4, designed to tolerate a wide range of hydraulic inputs.

- Input Constraints: The system is optimized for 125 bar. While the carrier relief can be set higher (170-220 bar), the breaker's internal regulation limits working pressure to protect components.

- Flow Tolerance: The breaker accepts 55–100 L/min. It uses an internal flow regulator to bypass excess flow, ensuring the impact frequency does not exceed the structural limit of 1,550 bpm.

- Fluid Requirements: Mineral-based hydraulic oil is standard, with viscosity limits typically between 32 and 46 cSt at operating temperature.3 The system is also compatible with biological lubricants.5

4.2 Domain B: Energy Release (Valve Cycling)

The energy release is controlled by the Distributor (Part No. 86639093) housed in a Distribution Box (Part No. 86639085).11

- Type: Spool valve, hydraulically piloted.

- Mechanism: The valve creates a bistable hydraulic multivibrator. The frequency of oscillation is determined by the flow rate and the stroke length (which is variable in some Montabert models, though the SC-36 is primarily a fixed-energy, variable-frequency system via flow).

- Protection: The valve logic includes the Anti-Blank Firing interlock. The hydraulic circuit effectively "neutralizes" if the tool is fully extended, preventing the build-up of high pressure in the upper chamber.1

4.3 Domain C: Kinematic Constraint (Guidance)

The kinematic path of the piston and tool is strictly constrained to ensure axial impact.

- Piston Guidance: The piston (Part No. 86639077) runs directly in the bored/honed Monobloc body.11 The absence of a separate cylinder liner improves heat transfer and structural rigidity.

- Tool Guidance: The tool (76 mm dia) is supported by a Bushing (Part No. 86639135).11 In the SC series, this is often a single long composite or bronze bushing.

- Rotation Prevention: The tool shank usually features an oval or flattened retainer slot profile to prevent rotation during prying maneuvers, though prying is discouraged. The Silver Clip (retention clip) engages these slots to retain the tool while allowing axial "float" for the reset stroke.

4.4 Domain D: Force Delivery (Impact Interface)

The interface is the flat contact surface between the piston nose and the tool shank.

- Impact Geometry: The piston diameter is matched to the 76 mm tool.

- Tool Metallurgy: The tool steels (Chisel, Blunt, Pyramidal) are heat-treated alloy steels designed for high surface hardness (wear resistance) and core toughness (shock absorption).

- Stress Wave Management: The length of the piston ($L_{piston}$) determines the wavelength of the stress wave ($\lambda = 2L_{piston}$). This wavelength must be tuned to the rock type; shorter waves (shorter pistons) are better for hard rock, while longer waves transfer energy more deeply into soft material.

4.5 Domain E: Sealing and Containment

Effective containment of high-pressure fluid and nitrogen gas is critical.

- Hydraulic Sealing: The piston is sealed by a specialized Seal Kit (Part No. 86639903).12 This kit typically includes:

- U-Cup Seals (PU): For dynamic sealing of the reciprocating piston (Part No. 86639242, 86639234).

- Buffer Rings (PTFE): To protect the main seals from pressure spikes.

- Wiper/Scraper Seals: To clean the tool shank as it retracts, preventing contamination.

- Gas Sealing: The accumulator uses a Diaphragm (Part No. 86639952) 12 made of highly resilient elastomer to separate N2 from oil. The integrity of this diaphragm is the single most critical factor for the breaker's smooth operation.

4.6 Domain F: Control Interface

The SC-36 is designed for autonomy.

- Operator Control: The operator controls only the On/Off state via the carrier's auxiliary circuit pedal/button.

- Automatic Regulation: The breaker features an internal Automatic Pressure Regulator.1 This device creates a constant operating pressure (approx. 125 bar) regardless of the carrier's input flow (as long as it is within the 55-100 L/min range). If the carrier supplies excessive flow, the regulator bypasses it; if the oil heats up and viscosity drops, the regulator adjusts to maintain peak blow energy.

- Adaptation: While the impact energy is relatively constant (governed by the regulator), the frequency varies with flow. This allows the operator to "tune" the breaking speed by modulating engine RPM or flow settings.

5. L3 & L4: Mechanisms and Components Decomposition

This section breaks down the major assemblies into their constituent components, utilizing the extensive parts list data extracted from service manuals.8

5.1 Power Cell Assembly

The power cell is the heart of the machine, comprising the hydraulic work components.

- Cylinder (Body): Part No. 86639044. A massive, single-piece casting. It contains the high-pressure and return galleries, the pilot ports, and the main bore.

- Strike Piston: Part No. 86639077.

- Dimensions: Length approx. 400-500mm (estimated from tool length comparisons), Diameter 76-77mm.

- Mass: Estimated at 20-25 kg based on breaker weight class ratios.

- Function: Converts hydraulic pressure to kinetic energy.

- Distributor Valve Assembly:

- Distributor: Part No. 86639093.

- Distribution Box: Part No. 86639085.

- Distribution Cover: Part No. 86639101.

- Valve: Part No. 86639119 (Pressure Regulator Valve).

- Function: Controls timing and pressure modulation.

5.2 Accumulator System

Located at the top of the breaker, integrated into the monobloc design.

- Diaphragm: Part No. 86639952 (also referred to as Membrane).

- Upper Flange: Part No. 86639028.

- Lower Flange: Part No. 86639010.

- Charging Valve: A standard gas valve for N2 pre-charging.

- Function: Stores energy on the upstroke, releases on downstroke, dampens pulsations.

5.3 Tool Retention and Guidance (Lower Cell)

- Bushing: Part No. 86639135. A wear part that must be greased regularly.

- Retaining Pin (Silver Clip): Part No. 86693801.

- Locking Mechanism: The "Silver Clip" system allows the retaining pin to be removed easily with a multi-tool, facilitating rapid tool changes on-site without heavy sledgehammers.

- Tool: Standard diameter 76mm. Available as Moil (86639226), Chisel (86639218), or Blunt (86641438).

5.4 Housing and Suspension

The power cell acts inside a protective cradle.

- Housing (Cradle): Part No. 86693785.11 Fabricated steel box.

- Upper Dampener: Part No. 86644762. Elastomeric block to absorb recoil.

- Lower Dampener: Part No. 86644770. Polyurethane buffer to center the cell and absorb axial shock.

- Wear Plates (Slippers): Plastic guiding slippers (e.g., Part 86726726 in related models) prevent metal-to-metal contact between the vibrating power cell and the static housing.

6. L5: Quantitative Parameter Specification

This section presents the extracted and calculated data in tabular format for direct use in the SEDF.

6.1 System Specifications

| Parameter | Value | Units | Normalized Value | Source |
| --- | --- | --- | --- | --- |
| Model | SC-36 | - | - | 3 |
| Operating Weight | 365 | kg | 365 kg | 3 |
| Carrier Weight | 4 – 10 | tons | 4,000 – 10,000 kg | 3 |
| Total Length | 1,379 | mm | 1.379 m | 2 |
| Width | 240 | mm | 0.24 m | 16 |
| Height (Cradle/Tool) | 1,530 | mm | 1.53 m | 16 |
| Noise Level (LwA) | 124 | dB | - | 17 |

6.2 Hydraulic Circuit Parameters

| Parameter | Value | Units | Normalized Value | Source |
| --- | --- | --- | --- | --- |
| Operating Pressure | 125 | bar | 12.5 MPa | 3 |
| Flow Range (Min) | 55 | L/min | 0.92 L/s | 3 |
| Flow Range (Max) | 100 | L/min | 1.67 L/s | 3 |
| Max Back Pressure | 20 | bar | 2.0 MPa | 2 |
| Relief Valve Setting | 170 – 220 | bar | 17 – 22 MPa | 3 |
| Input Power | 21 | kW | 21,000 W | 2 |
| Oil Temp (Optimum) | 46 – 71 | °C | 319 – 344 K | 3 |
| Oil Viscosity | 32 | cSt | 32 mm²/s | 3 |

Insight: The operating pressure of 125 bar is relatively low compared to heavy breakers (often 160-180 bar). This lower pressure reduces seal friction and heat generation, contributing to the system's longevity and suitability for smaller carriers with limited cooling capacity. The strict back pressure limit of 20 bar 18 is crucial; exceeding this can damage the diaphragm and reduce impact efficiency by resisting the piston's return stroke.

6.3 Impact Performance Metrics

| Parameter | Value | Units | Normalized Value | Source |
| --- | --- | --- | --- | --- |
| Impact Frequency | 1,550 | bpm | 25.8 Hz | 2 |
| Energy Class | 1,000 | ft-lb | 1,355 J | 3 |
| Tool Diameter | 76 | mm | 0.076 m | 3 |
| Working Length (Tool) | 400 | mm | 0.40 m | 2 |
| Piston Stroke | ~60-80 | mm | 0.06-0.08 m | Est. (High Freq) |

Insight on Impact Energy: There is a critical distinction between the "Energy Class" (1,000 ft-lb / 1,355 J) used for marketing and the thermodynamic reality.

Calculation:

$$Power (W) = Energy/Blow (J) \times Frequency (Hz)$$

$$21,000 W = E_{blow} \times 25.83 Hz$$

$$E_{blow} = 21,000 / 25.83 \approx 813 \text{ Joules}$$

Assuming a hydraulic efficiency of 80-90%, the actual impact energy delivered to the rock is likely in the range of 650 – 750 Joules per blow. The "1,000 ft-lb" class is a comparative rating (CIMA/AEM standard) indicating it does the work equivalent of a 1,000 lb drop ball, not that it possesses 1,355 J of kinetic energy. For engineering simulation, the 750 J figure should be used.

6.4 Accumulator & Gas Specifications

| Parameter | Value | Units | Normalized Value | Source |
| --- | --- | --- | --- | --- |
| Type | Diaphragm | - | - | 11 |
| Gas | Nitrogen | N2 | - | 19 |
| Volume | ~0.5 - 1.0 | L | - | Est. |
| Pre-charge Pressure | 40-50 | bar | 4-5 MPa | Inferred 20 |

Insight: The pre-charge pressure is a critical tuning parameter. Standard hydraulic practice suggests pre-charging the accumulator to approximately 40-60% of the working pressure. With a working pressure of 125 bar, a pre-charge of 50 bar ensures the diaphragm does not bottom out during operation but still provides adequate energy storage. Some sources 21 for similar sized breakers suggest ~50 bar.

7. Hydraulic and Thermodynamic Performance Analysis

7.1 Efficiency and Heat Generation

The SC-36 operates at a high frequency (25.8 Hz). High-frequency cycling generates significant heat due to fluid friction through the distributor valve and seal friction on the piston.

- Heat Dissipation: The breaker relies on the carrier's oil cooler. With an input power of 21 kW, and assuming 70% efficiency (typical for percussive tools), approximately 6.3 kW of heat is rejected into the oil. The specification of an optimum temperature range (46-71°C) 3 is vital; exceeding 80°C leads to seal degradation and a drop in oil viscosity (below 32 cSt), which can cause internal leakage and loss of impact power.

7.2 The Role of the Automatic Pressure Regulator

A standout feature of the SC-36 is the Automatic Pressure Regulator.13 In standard breakers, impact energy fluctuates with the carrier's pump flow. If flow drops, pressure often drops, weakening the blow.

- Mechanism: The Montabert regulator creates a variable restriction or bypass. If the input flow is low (e.g., 60 L/min), the regulator maintains the system pressure at 125 bar, ensuring the piston still accelerates to full velocity, albeit at a lower frequency.

- Benefit: This decouples "Impact Energy" (Force) from "Frequency" (Speed). Every blow delivered is a "full power" blow, regardless of engine RPM. This is crucial for breaking hard rock where a weak blow merely bounces off (elastic deformation) without causing fracture.

7.3 Energy Recovery System

The SC-36 captures recoil energy.4

- Physics: When the piston strikes a rock that does not break instantly, the rock acts as a spring, rebounding the piston upward.

- Hydraulics: In a basic breaker, this rebound forces oil out to the tank. In the SC-36, the valve geometry traps this high-pressure rebound fluid in the lifting chamber. This captured pressure assists the hydraulic supply in lifting the piston for the next stroke.

- Result: The system requires less flow from the pump to lift the piston, allowing the saved flow to be used to cycle the valve faster. This explains how the SC-36 achieves 1,550 bpm—a frequency 20-30% higher than many competitors in the 350 kg class (who often run at 1,000-1,200 bpm).16

8. Operational Limits and Failure Engineering

8.1 Blank Firing (Dry Firing)

"Blank firing" occurs when the piston cycles without the tool pressed against rock. The piston strikes the retaining pin or front head instead of the tool.

- Consequence: The shock wave is not absorbed by rock but reflects into the breaker body. This causes catastrophic failure: cracked housings, sheared retaining pins, and damaged side bolts.

- SC-36 Solution: The Hydraulic Interlock.1 When the tool drops (disengages), it opens a bypass port. The high-pressure supply flows directly to the return line without building pressure in the piston chamber. The piston stops cycling immediately. This feature significantly extends the life of the monobloc body and tool retainers.

8.2 Seal and Diaphragm Wear

- Diaphragm: The diaphragm is the "fuse" of the accumulator. Over time, N2 diffuses through the elastomer, lowering pre-charge. If the pre-charge drops too low, the diaphragm deforms excessively, hitting the gas valve or oil port, leading to rupture. A ruptured diaphragm fills the hydraulic circuit with N2 gas (aeration), causing "spongy" performance and cavitation damage to the pump.

- Maintenance: The SC-36 manual recommends checking the diaphragm and N2 pressure annually. The "maintenance-free" claim refers to the lack of routine charging, but replacement is eventually required.22

8.3 Back Pressure Sensitivity

The SC-36 is sensitive to back pressure (limit 20 bar).

- Reason: High back pressure acts on the top of the piston during the return stroke, fighting the lifting force. This reduces the stroke length and impact velocity. It also creates a "hydraulic lock" effect on the diaphragm, preventing it from discharging fully. This is why the return line must be sized correctly (at least 1/2" to 3/4") and routed directly to the tank, bypassing the main control valve if possible.3

9. Comparative Analysis: SC-36 vs. Competitors

To contextualize the SC-36 specifications, it is useful to compare it with the standard "Nitrogen-Fired" (Gas-Assist) breaker architecture (e.g., NPK or some Soosan models).

| Feature | Montabert SC-36 (Fully Hydraulic) | Typical Nitrogen-Fired Breaker |
| --- | --- | --- |
| Piston Actuation | Hydraulic pressure drives downstroke | Compressed N2 gas drives downstroke |
| Accumulator | Diaphragm (small, cushions pulses) | Piston-type (large, stores drive energy) |
| Linearity | Constant blow energy (Regulated) | Energy varies with gas temp/pressure |
| Frequency | High (1,550 bpm) | Medium (800-1,200 bpm) |
| Back Pressure | Sensitive (Max 20 bar) | More tolerant (Max 25-30 bar) |
| Efficiency | High (Energy Recovery) | Lower (Heat loss in gas compression) |
| Impact Characteristic | High Freq / Moderate Energy | Low Freq / High Energy |

Implication: The SC-36 is engineered for Productivity in Medium Material. Its high frequency makes it a "machine gun" compared to the "cannon" of a gas-fired breaker. It rapidly creates micro-cracks in concrete or stratified rock. However, on extremely hard, massive granite boulders, a gas-fired breaker with a heavier, slower piston might achieve fracture initiation more easily due to higher momentum per blow. The SC-36 compensates for this with the energy recovery system, allowing it to "drill" through hard spots with rapid-fire impacts.

10. Conclusion

The Montabert SC-36 Hydraulic Rock Breaker represents a highly optimized implementation of the fully hydraulic, monobloc architecture. Its engineering prioritizes structural rigidity (by eliminating tie rods), operational consistency (via automatic pressure regulation), and thermodynamic efficiency (through energy recovery).

For the purpose of the Standardized Engineering Decomposition Framework (SEDF), the SC-36 should be characterized as:

- Level 0: A hydraulic-to-kinetic energy transducer utilizing stress wave propagation.

- Level 1: A regenerative hydraulic circuit with pneumatic buffering.

- Level 5 (Key Simulation Parameters):

- Mass: 365 kg

- Impact Energy: ~750 Joules (Actual)

- Frequency: 25.8 Hz (1,550 bpm)

- Hydraulic Input: 125 bar @ 55-100 L/min

This specification provides a robust basis for the "System B" input in the proposed synthesis experiment, offering a distinct contrast to pneumatic or electromechanical impact systems in terms of power density and control logic. The gathered data suggests that any simulation must account for the breaker's frequency-dependent power output and the non-linear behavior introduced by the accumulator and regeneration valve.

Works cited

- SD36 Hydraulic Rock Breaker: quiet, powerful & robust - Montabert, accessed January 20, 2026, https://www.montabert.com/en/product/sd36-hydraulic-rock-breaker/

- Montabert SC-36 - EMS Construction Machinery Dublin Ireland, accessed January 20, 2026, http://www.emsmachinery.com/montabert-rock-breakers/2493-montabert-sc-36.html

- Compact Hydraulic Breakers - Montabert USA, accessed January 20, 2026, https://montabertusa.com/products/compact-demolition/

- Hydraulic Rock Breaker - Inmalo, accessed January 20, 2026, https://www.inmalo.co.uk/attachments/articles/a44_EN-SCrangeoverview-a%20(02-2016).pdf

- compact range - Montabert USA, accessed January 20, 2026, https://montabertusa.com/wp-content/uploads/2016/04/Range_SC_US_03.2021.pdf

- BREAKERS - Parameter1, accessed January 20, 2026, https://cdn.base.parameter1.com/files/base/acbm/fcp/document/2018/08/Montabert_Hydraulic_Breaker_Full_Line.5b6b5f906efce.pdf

- Hydraulic Rock Breaker Montabert | PDF | Energy And Resource | Nature - Scribd, accessed January 20, 2026, https://www.scribd.com/document/436781031/Hydraulic-Rock-Breaker-Montabert

- EN Spare Parts Manual | PDF | Equipment | Mechanical Engineering - Scribd, accessed January 20, 2026, https://www.scribd.com/document/486375556/XL-1300-2012

- NDA Column Debuts - Professional Demolition, accessed January 20, 2026, http://pdamericas.com/wwwdocs/_sites_media/170731-170/uploads/dokument/Magazines/2019/PDa%203%202019.pdf

- Hydraulic Rock Breaker - Worsley Plant, accessed January 20, 2026, https://www.worsleyplant.co.uk/app/uploads/2018/09/Montabert-Variable-Range-Brochure.pdf

- Manual SC36 | PDF | Piston | Engine Technology - Scribd, accessed January 20, 2026, https://www.scribd.com/document/894135490/Manual-SC36

- Montabert SC36 Seal Kit 86639903 Diaphragm 86639952 Membrane 86639960 Maintenance Kits - cero machinery co., limited, accessed January 20, 2026, https://www.cero.top/product-montabert-sc36-86639903-seal-kit-86639952-diaphragm.html

- Midwest 9, April 27, 2024 by Construction Equipment Guide - Issuu, accessed January 20, 2026, https://issuu.com/constructionequipmentguide/docs/mw092024

- Rompedor 86776267-01 - SC28 | PDF | Valve | Piston - Scribd, accessed January 20, 2026, https://www.scribd.com/document/704299104/Rompedor-86776267-01-SC28

- Montabert SC36 Specifications & Technical Data (2006-2021) | LECTURA Specs, accessed January 20, 2026, https://www.lectura-specs.com/en/model/construction-machinery/attachments-hydraulic-breakers-montabert/sc36-1032374

- Blue Line Hydraulic Breakers | Product Range, accessed January 20, 2026, http://3.imimg.com/data3/XO/XO/MY-2719573/hydraulic-breaker.pdf

- www.montabert.com - Six Equipment, accessed January 20, 2026, https://www.sixequipment.be/wp-content/themes/sixequipment/images/location/2001-1-m.pdf

- Montabert Hydraulic Breakers, accessed January 20, 2026, http://3.imimg.com/data3/RY/CX/MY-2719573/hydraulic-breaker.pdf

- How to Charge Accumulators with Nitrogen - Quality Hydraulics & Pneumatics, Inc., accessed January 20, 2026, https://qualityhydraulics.com/blog/accumulators/how-charge-accumulators-nitrogen

- How to Calculate Accumulator Pre Charge - YouTube, accessed January 20, 2026, https://www.youtube.com/watch?v=mEs5PecXOT8

- H1DROMEK - Hidromek.kz, accessed January 20, 2026, https://hidromek.kz/upload/information_system_16/2/5/4/item_254/102B%20MAESTRO%20(JD-ZF)%20YENI%20FORMAT-22122014.pdf

- MAINTENANCE INSTRUCTIONS BLADDER ACCUMULATORS - PacSeal Hydraulics, accessed January 20, 2026, https://www.pacsealhydraulics.com/wp-content/uploads/2020/10/Accumulator-Maintenance-Instructions-Rev-B.pdf